Disposable Oxygen Hoods/Tents

Part number 231001 to 23100134

Assembly and General Instructions:

1) Remove oxygen hood/tent from the packaging.

2) Unfold oxygen hood/tent and remove press-out discs from the 22mm tubing ports where present. In certain hoods/tents such as the Double-Tent Outer, the discs have been left in place. In other models they have already been removed.

3) Snap the back panel into the body section using all the snap connectors. Note: If the tent is a No-Bottom, NICU or Partial Bottom design, the snap connections on the bottom edge of the back panel are not used.

4) The three 22mm tubing ports in the back panel of the Outer Double-Tent are available for vacuum and aerosol/humidity tubing. Note: Tubing, connectors, and filters of the user’s choice will need to be added.

5) Assembled hoods/tents should be used according to the hospital’s protocols. 

Note: Bedding should be placed between the patient and the hood/tent bottom where it exists.

CAUTION: Prior to placing the patient inside the hoods/tent, the hood/tent should be inspected to ensure that the press-out discs have been removed from the hood/tent and the hood/tent area. Ensure that all the snap connections are in working order.

CAUTION: No smoking, no use of electrical appliances, no open flames should be allowed when oxygen is in use; only approved toys should be given to the patient.
CE0086

V1.1

22 march 2007
