VM3COP40.72 Production procedure for 0110238 Oxygen sensor (R-22AT)

| Parts list: |  |  |
| --- | --- | --- |
| Qty | Description | Part No. |
| 1 | Oxygen sensor - Automotive R-22AV | 0110132 |
| 1 | 2 pin Molex connector |  |
| 2 | Molex pin |  |
| 1 | Sensor adapter |  |
| 1 | Sensor label |  |

Tool list:

Soldering iron

Wire cutters

Stanley knife

Sensor cap removal tool

Super glue (Loctite 435)

| - Remove the sensor from the sealed protective bag. |  |
| --- | --- |
| - Remove the top cap from the sensor. |  |
| - Remove the white plastic surround from the Molex connector. |  |
| - Using a soldering iron and pliers, remove the pins. |  |
| - Cut lengths of black and red wire to 75mm. - Strip 3mm from each end of each wire. - Tin one end of each wire. - Apply a Molex crimp pin to the non-tinned wire. |  |
| - Insert the wires, as shown, into the Molex connector. |  |
| - Solder the wires to the solder points as shown. |  |
| - Prepare the foam cap by placing double sided sticky tape to one side. - Peel away the backing from the double sided sticky tape. |  |
| - Place the foam cap, sticky side down, onto the top of the sensor and gently press into place. |  |
| - Noting the serial number, remove the sensor label. |  |
| - Apply the new label, ensuring that the serial number matches the one previously removed. |  |
| - Remove the black O ring. |  |
| - Place a drop of super glue onto the thread. |  |
| - Attach the sensor adapter to the sensor thread. |  |
| - Place the O ring over the sensor adapter thread. - The sensor is now ready for testing. |  |