

September 10, 1994


Nigel Harrison

Northern Hi-Tec
4th March 2005

Dear  Nigel,
Enclosed are the faulty boards.  The faults are:

1
C9 ground pad not connected,


IC6 ground pad not connected,


CN2 right hand fixing pad not connected to ground,


CN3 left fixing pad not connected to both grounds,

CN3 right hand fixing pad not connected to ground,

2
solderability is not good,

3
some tinning material extends off the solder pads,

4
on the some vias, the drill hole is very close to the edge of the pad, not central,

5
the rear silk screening is poor; the front screening is a bit better.

I hope that this is helpful.

Best wishes,

Peter Anderson



(  Page 2

