[image: image1.jpg]ARAB HEALTH
2002

Summary of products

Pulse Oximetry Sensors Viamed  Dolphin  Aristo
Finger sensors v v v
Multi-site 'Y’ sensors v v v
Ear sensors v
Extension cables v v v
Adapter cables v v v
D.O.T. Sensors v
Wrap sensors v
Disposables v v ~

Pulse oximetry sensor repair services
Pulse oximeters - Stand alone, hand held, compact PC
Pulse oximetry simulators and testers

Oxygen sensors
Oxygen monitors and analysers

Oxygen hoods
Patient monitors
Infant resuscitation cabinets

Infant resuscitators
Radiant warmers

Phototherapy masks
Phototherapy light shields

ECG cables

Temperature probes

Supra-maximal nerve stimulator

Chart & recorder paper [
VIARMED

Imail infeid




[image: image2.png]



        User Instructions     

                                                    Pt No                                

Head Boxes

	


Adjust the height of the neck aperture (A) to reduce excessive gas an heat loss allowint the infants neck freedom of movement. Tighten screws

Use  the access cutouts in the base (B) to run IV Lines  or cxygen sensors cables etc. into the Head box.

The 22mm input hole (C) can be used to supply Oxygen via a standard oxygen tube 5mm or a humidified gas using 22mm or 15mm standard fittings
Warning

Whilst using all types of  Viamed Head boxes routinely check blood oxygen level

To be certain of adequate carbon dioxide washout, use at least 6 litres a minute of air/oxygen mixture.

For babies weighing less than 1500  grams. the Viamed Headbox should be pre-warmed to approximately 35 C before placing over the inland. Either in the incubator or under a radiant warmer, Placing a cold headbox over a small baby may decrease body temoerature as radiant losses from the baby can contribute to the warming of the oxygen hood.

It is recommended that the gas mixture be warmed and humidified before introduction.

Oxygen sensors should ideally be placed as near to the infants mouth as possible

Accessories 

PP1968  Oxygen connector

PP1969    Divertor

PP1970 special divertor

              Nascor Divertor

              Nascor soft neck seal 

              Oxygen Analyser

ViamedLtd   15 Station Road Crosshills, Keighley West Yorkshire BD20 7DT

+44 1535 634542  Info@Viamed.co.uk  Fax +44 1535 635582

User Instructions Head boxes
J S Lamb     Page 2
30/08/02

[image: image1.jpg][image: image2.png]