VM3COP50.20 Tom Thumb Prebuild Component Inspection

| Tool list: |  |
| --- | --- |
| Isopropyl alcohol |  |
| Lint free cleaning cloth |  |
| Inspection light |  |

| Adjustable valve |  |
| --- | --- |
| Part number and description: | Inspection: |
| 0330223 - Adjustable valve spring | Ensure the spring clean |
| 0330225 - Adjustable valve seating | Ensure the valve seating is clean and inspect the threads for burrs. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330227 - Adjustable valve seat | Ensure the valve seat clean |
| 0330229 - Adjustable valve body | Ensure the valve body is clean. Inspect each of the threads for burrs. Inspect each port is free of foreign material. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330230 - Adjustable valve screw | Ensure the valve screw is clean and inspect the threads for burrs. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330231 - Adjustable valve internal stop | Ensure the valve internal stop is clean and inspect the threads for burrs. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330232 - Adjustable valve collar | Ensure the valve collar is clean and inspect the threads for burrs. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330233 - Adjustable valve centre screw | Ensure the valve centre screw is clean. |
| 0330234 - Adjustable valve side screw | Ensure the valve side screw is clean. |

| Precision valve |  |
| --- | --- |
| Part number and description: | Inspection: |
| 0330222 - Precision valve spring | Ensure the precision valve spring is clean. |
| 0330224 - Precision valve body | Ensure that the precision valve body is clean. Inspect the threads for burrs. Inspect each of the ports for obstruction. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330225 - Precision valve seating | Ensure the precision valve seating is clean. Inspect the thread for burrs. Inspect the port for obstruction. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330227 - Precision valve seat | Ensure the precision valve seat is clean. |
| 0330228 - Precision valve adjustment screw | Ensure the valve adjustment screw is clean and inspect the thread for burrs. Inspect the chroming for blemishes, discolouration and peeling. |

| Tom Thumb body, ports and bolts |  |
| --- | --- |
| Part number and description: | Inspection: |
| 0330200 - TT460 main body block 0330201 - TT480/490 main body block 0330202 - TT495 main body block | Ensure the body block is clean. Inspect each thread for burrs. Inspect each port is free of foreign material. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330205 - Flow meter bolt | Ensure the flow meter bolt is clean. Inspect the thread for burrs. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330206 - Blocking bolt and nut | Ensure the blocking bolt and nut are clean. Ensure the nut can be rotated on the bolt thread. Inspect the threads for burrs. Inspect the ports for obstruction. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330207 - Blanking bolt | Ensure the blanking bolt is clean. Inspect the thread for burrs. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330208 - Inlet 10mm | Ensure the inlet is clean. Inspect the thread for burrs. Inspect the port for obstruction. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330209 - Outlet 15mm | Ensure the outlet is clean. Inspect the thread for burrs. Inspect the port for obstruction. Inspect the chroming for blemishes, discolouration and peeling. |
| 0330235 - Inlet restrictor | Ensure the inlet restrictor is clean. |
| 0330296 - Blanking bolt | Ensure the blanking bolt is clean. Inspect the thread for burrs. Inspect the chroming for blemishes, discolouration and peeling. |