	VANDAGRAPH
  COMPANY OPERATING PROCEDURE

	Oxygen Sensor manufacture

	Modification of Sensor case

VM3/COP/40.11

	Date: 30-Sep-03
	Revision Date: 4-Jul-11
	Issue 1

	Only to be attempted by trained personel



	Parts Required

	Quantity
	Description
	Part No.
	

	1
	R22 Any version can be used but R-22D is preferred
	
	


Method.

1. Open the packaging and remove the O2 sensor

2. Check the O2 sensor for damage and signs of leaking electrolyte.

3. Remove the O2 sensor specification leaflet and discard. 

4. Remove excess outer case unto level of PCB.

a. [image: image1.jpg]LR



[image: image2.jpg]


 Insert Flow divertor into the Drill chuck 
There is a brass equivalent to the flow divertor

b. Screw the sensor onto the Flow divertor

c. Switch on the drill and ensure the sensor rotates vertically and evenly

d. [image: image3.jpg]At | .



Switch off the drill





e. Lower the chuck so that the top of the vice jaws are level with the top of the PCB

f. [image: image4.jpg]


The chuck jaws should not grip the sensor. They are used as a guide only and prevent any lateral movement during removal




g. Switch on the drill

h. Using a sharp blade gently apply pressure just above the PCB until the casing parts

i. Ensure the PCB is still attached to the housing

j. [image: image5.jpg]


If it has parted glue back together

Case modificartion.doc
Page 1
7/4/2011

