VALVE SELECTION

TOMSPAVE is a keyword used in valve selection, please use as follows;

Type – 2-way, 3-way etc.

Operation – Normally open, closed etc.

Media – air, water etc.

Size – ¼”, ½” etc.

Pressure – 10 bar, 20 bar etc.

Ambient – Temperature in the area where the valve is located

Voltage – 110/50, 24DC etc.

Enclosure – IP65, Atex approved, Waterproof etc.

Helpful Tips on Valve Selection

- If media other than general service (air, inert gas, water, light oil) is required, check the Fluid Resistance Guide in the catalogue.

- If no flow rate is requested, select a valve with the largest orifice for pipe size nearest the required pressure.

- Be aware of the different pressure ratings for AC and DC.

- Be aware of the minimum operating pressure differential on certain valves.