Medical Device Directive (MDD) 93/42/EEC                     Packaging Procedure            
Company Name: Marshall Products Limited            Product: Neonatal Resuscitation Circuit

Packaging Procedure:
1. ‘Pack’
The screw cap on the NeoPEEP is screwed down fully and then loosened by one turn before packaging. The NeoPEEP™ Single Use Resuscitation Circuit is individually wrapped in clear plastic heat sealed bags.

They do not need to be sterile, but must be ‘clinically clean’.

A Product label is to be stuck on to the outside the plastic bag with the ‘pack’ section ticked 

The dimensions of the NeoPEEP™ Single Use Resuscitation Circuit in the sealed plastic bags will be:



30cms X 20cms

2. ‘Inner Box’

The single pack of the NeoPEEP™ Single Use Resuscitation Circuit then has to be placed into a box in 2 rows of 10 side by side. The approximate size of the box should be:



36cms X 27cms X 15cms
Inside of this box there should be 1 X Instructions for Use. 
3. ‘Outer Case’
The ‘Outer Case’ should contain 20 boxes of ‘inner boxes’ totalling 200 NeoPEEP™ Single Use Resuscitation Circuits per case.

A Product Label is to be stuck on the outside of this box with the ‘Outer Case’ box ticked.  

There should be the standard symbols displayed on the outer case as in all Marshall Products cases have at present :-

[image: image4.emf]
[image: image1.png]




 INCLUDEPICTURE "http://www.technologystudent.com/images4/frag1.gif" \* MERGEFORMATINET [image: image2.png]&





 INCLUDEPICTURE "http://www.technologystudent.com/images4/uptop.gif" \* MERGEFORMATINET [image: image3.png]<
<





P/O
Product Name
N.W



XX Kgs


G.W



XX Kgs


Product Code:

XXXXXX


Carton Number:

XX

Manufactured by: Marshall Products Limited, 1 The Maltings, Brassmill

Lane, Bath, BA1 3JL, United Kingdom 

tel: 0044 845 612 8888 fax: 0044 845 612 8889
Batch numbers are allocated as follows:- 

 

The batch number is edited according to monthly quantity manufactured. Every 1,000 pieces carry the same batch number, the number changes for each 1,000 pieces manufactured each month and also will change for each change of size and type of NeoPEEP manufactured.

 

Example: October 2003 order, NeoPEEP Blue = 1,500 pcs and NeoPEEP Pink = 1,800 pcs. 

 

  Lot No.:   

L 0  3  1  0  0  1 (NeoPEEP Blue the first 1,000 pcs)

 
L 0  3  1  0  0  2 (NeoPEEP Blue the balance 500 pcs)

L 0  3  1  0  0  3 (NeoPEEP Pink the first 1,000 pcs)

 
L 0  3  1  0  0  4 (NeoPEEP Pink the balance 800 pcs)

         

a. The first letter indicates that the product is manufactured at Three Lions.

b. The 1st to 2nd numbers indicate the year the products were manufactured.  

c. The 3rd and the 4th numbers indicate the month the products were manufactured.             

d. The 5th and the 6th numbers indicate the batch of 1000 in which the products were manufactured. Numbering changes for each 1,000 pieces, increasing sequentially through the month.   



































































�


  0434








Issue C

                                                                                                                                                                                                                                        


