Specification Datasheet

Viamed

SpiroTrue H

OEM Disposable Single Patient Adult Flow Sensor

CE

| ## Part numbers | 4310003 pack of 6 4310013 OEM pack of 20 |
| --- | --- |
| ## Model | SpiroTrue H – OEM disposable single patient adult flow sensor + tubing set |
| Primary packaging | 4310003 Box of 6 sets 4310013 PE pouch (bag) with 20 sets includes flow sensor with double PVC (DEHP free) tube connected plus adapter and hook |

#### Technical Data

| Measuring principle | Differential pressure measurement |
| --- | --- |
| Reproducibility | ≤ 10% at flow range 0 to 100l/min |
| Designed in accordance with | 93/42/EEC (Annex I and Annex II) |
| Housing material | ABS (Acrylnitril-Butadien-Styrol) |
| Weight | ≤ 9.5g (without tubing) |
| Dead space | ≤ 10ml |
| Connectors | 15mm O.D./15mm O.D. in accordance with ISO 5356-1 |
| Conforms to | EN ISO 10993-5 |
| Packaging | Non-sterile |
| Recommended storage temperature | # -20ºC to 50ºC |
| Humidity (R.H.) | 5% to 95% |
| Storage period | ≤ 3 years (unopened, away from direct sunlight) |
| Operating temperature | 10ºC to 40ºC |
| Relative humidity | 5 to 95%, non-condensing |
| Notes: Before using the sensor consult the appropriate ventilator user instructions for more detailed information. SpiroTrue H is compatible with Hamilton single use flow sensor part number 279331 |  |