Specification Datasheet

Viamed

SpiroTrue PED

OEM Disposable Single Patient Paediatric Flow Sensor

CE

| ## Part number | 4310070 OEM pack of 6 |
| --- | --- |
| ## Model | SpiroTrue PED – OEM disposable single patient paediatric flow sensor |
| Primary packaging | PE pouch (bag) with 6 sets |

#### Technical Data

| Measuring principle | Differential pressure measurement |
| --- | --- |
| Reproducibility | ≤ 10% at flow range 0 to 60l/min |
| Designed in accordance with | 93/42/EEC (Annex I and Annex II) |
| Housing material | ABS (Acrylnitril-Butadien-Styrol) |
| Weight | ≤ 6.5g (without tubing) |
| Dead space | ≤ 7ml |
| Connectors | 15mm O.D./15mm O.D. in accordance with DIN EN ISO 5356-1 |
| Conforms with | EN ISO 10993-5 |
| Components including tubing conform with | 2011/65/EC (RoHS) |
| Packaging | Non-sterile |
| Storage temperature | # -20ºC to 50ºC |
| Humidity (R.H.) | 5% to 95% |
| Storage period | ≤ 3 years (unopened, away from direct sunlight) |
| Operating temperature | 10ºC to 40ºC |
| Relative humidity | ≤ 95%, non-condensing |
| Warranty period | ??12 months from date of sales invoice |
| Notes: Before using the sensor consult the appropriate ventilator user instructions for more detailed information Sensor was tested on OxyLog 3000 SW1.21 Setup: 190 Dräger circuit REF 5704964-10; 2.0l Dräger test lung with restrictor 7mm: REF 2165694 SpiroTrue PED with double PVC (DEHP free) tube is available upon request |  |