Quick Start Guide for MD300-D Finger Pulse Oximeter

	This Quick Start Guide describes the steps to successfully start and use the instrument.  
Before placing the instrument in service please read the user manual that accompanies the device.


	[image: image7.png]




	[image: image1.jpg]193 77

FINGERTIP





[image: image4.jpg]



[image: image5.png]




	[image: image6.jpg]weQdg  wdayd
N~ N

N~
o |

Pulse Oximeter




Low power indicator

  [image: image2.jpg]SpO22% PR bpm

95 a1

Pulse Oximeter





SPO2 waveform




Battery Installation

Put the two AAA batteries into the battery cassette in the correct polarities.

Push the battery cover horizontally along the arrow shown as below:


[image: image3.png]



Notes: Battery polarities must be correctly installed. Otherwise, damage might be caused to the device.

Please put or remove batteries in the right order, or it is likely to damage the device bracket.

Please remove the batteries if the Oximeter will not be used for a long time

To Use the Oximeter

Nip the clamp as shown in the diagram.

Place one finger thoroughly into the rubber hole of the Oximeter.

Release the clamp.

When your finger is plugged into the Oximeter, your nail surface must be upward.


Switching the Oximeter on

Press the power switch button once on the front panel.

Ensure your finger does not tremble whilst the Oximeter is in use. 

Switching the Oximeter off

The Oximeter automatically powers off when no finger is in the unit for more than 8 seconds.

NEED TO KNOW HOW LONG IT TAKES BEFORE THE OXIMETER REGISTERS SPO2 READING.

Display Rotations

The Oximeter has 6 display rotations. Each time you press the power switch the Oximeter will rotate the display.
The final two displays of the Oximeter show the Pleth Waveform in two different formats; line trace or solid waveform.

Display Brightness Adjustment

When you press the power switch for more than a second, the brightness of the Oximeter will be changed by degrees, there are 10 levels of brightness; the default level is level four.

PR Bargraph





Power Switch





PR





SPO2




















