Quick Start Guide for MD300-D Finger Pulse Oximeter

	This Quick Start Guide describes the steps to successfully start and use the MD300-D.  

Before placing the MD300-D in service please read the user manual that accompanies the device.


	[image: image8.png]




	[image: image1.jpg]193 77

FINGERTIP





[image: image4.jpg]VIAMED




[image: image5.jpg]



PR = Pulse Rate
	[image: image6.png]


Low power indicator

[image: image7.jpg]weQdg  wdayd
N~ N

N~
o |

Pulse Oximeter




  [image: image2.jpg]SpO22% PR bpm

95 a1

Pulse Oximeter





SPO2 waveform




Battery Installation

Place the two AAA batteries into the battery cassette, observing the polarities.

Slide the battery cover horizontally along the case (as below):


[image: image3.png]



Note: Battery polarities must be correctly installed. Otherwise, damage maybe caused to the device.
Please remove the batteries if the Oximeter will not be used for a long time

To Use the Oximeter

Pinch the clamp, as shown in the diagram.

Place one finger, nail upward, thoroughly into the Oximeter.

Release the clamp.


Switching the Oximeter on

Press the power switch button once on the front panel.

Switching the Oximeter off

The Oximeter automatically powers off when a finger is no longer present for more than 8 seconds.

Display Rotations

The Oximeter has 6 display rotations. Each time you press the power switch the Oximeter will rotate the display.
The final two displays of the Oximeter show the Pleth Waveform in two different formats; line trace or solid waveform.

Display Brightness Adjustment

When you press the power switch for more than a second, the brightness of the Oximeter will be changed by degrees, there are 10 levels of brightness; the default level is level four.

PR Bargraph





Power Switch





PR





SPO2




















[image: image4.jpg]
Part No. 


