MD500-B
Technical Specification

	NIBP
	

	Measuring method
	Oscillometric method, automatic inflation, automatic measurement, 90 times memory

	Measuring range
	20mmHg ~ 300mmHg (2.7KPa ~ 40KPa)

	Measuring accuracy
	Within +/- 3mmHg (0.4KPa)

	Wrist cuff size
	310mm x 75mm

	Pressurization
	Automatic air inflation by an air pump

	Defilation
	Automatic exhaust

	Automatic power off
	To be automatically cut off after 3 minutes of non-use to save energy

	
	

	SpO2
	

	Range
	0% - 99%

	Resolution
	1%

	Accuracy
	80% - 99% +/- 2%

	
	70% - 80% +/- 3%

	
	0% - 69% unspecified

	
	

	Heart (Pulse) Rate
	

	Range
	30 - 235 bpm

	Resolution
	1bpm

	Accuracy
	+/- 2bpm or +/- 2%

	
	

	Alarm
	Low power indicator

	
	

	Display
	

	Type
	LCD

	Parameters
	NIBP, SpO2, PR, bargraph, SpO2 waveform

	Mode
	6 SpO2 display modes

	Brightness
	Adjustable 10 levels SpO2 screen

	
	

	Environmental
	

	Operating temperature
	5 - 40°C

	Operating humidity
	15% RH – 80% RH

	Storage humidity
	10% RH – 80% RH

	
	

	Mechanical
	

	Size
	105mm x 75mm x 34mm

	Weight
	120g (without batteries)

	
	

	Battery
	

	Type
	2 x AAA alkaline batteries

	Operation time
	About 30 hours for normal operation


