✅ Maxtec Blenders – MCQ Assessment (With References)



1. What is the main advantage of having a dual scale on a Maxtec DFB flow meter?

A. It allows two gases to be measured at once
B. It combines two flow ranges into one flow meter
C. It increases maximum pressure output
D. It removes the need for calibration

Answer: B
Source: Introduction to Maxtec Blenders, Page 1 – dual scale combines two flow ranges into one meter 



2. What feature of the BlenderBuddy 2 helps reduce waste gas?

A. Flow restrictor
B. Micro bleed
C. Pressure valve
D. Dual inlet system

Answer: B
Source: Introduction to Maxtec Blenders, Page 1 – “micro bleed… reduces the amount of waste gas” 



3. How does the BlenderBuddy 2 attach to a blender?

A. NIST hose connection
B. Quick-fit coupling
C. DISS inlet connection
D. Threaded oxygen port

Answer: C
Source: Introduction to Maxtec Blenders, Page 1 – attaches using DISS inlet connection 



4. What key benefit does the MaxBlend Lite provide when added to an existing blender?

A. Eliminates need for oxygen supply
B. Adds built-in oxygen monitoring with alarms
C. Converts low flow to high flow
D. Increases gas pressure automatically

Answer: B
Source: Introduction to Maxtec Blenders, Page 1 – built-in oxygen monitor with high/low alarms 



5. What type of display is included on the MaxBlend 2?

A. LED strip display
B. Backlit LCD display
C. Analogue dial
D. Touchscreen only

Answer: B
Source: Introduction to Maxtec Blenders, Page 2 – “backlit LCD” 



6. Which feature of the MaxBlend 2 helps reduce the risk of cross-contamination?

A. Built-in humidifier
B. Measuring oxygen away from the patient
C. High-pressure gas delivery
D. Dual gas input system

Answer: B
Source: Introduction to Maxtec Blenders, Page 2 – monitoring away from patient helps eliminate cross-contamination 



7. Which Maxtec product does NOT require piped medical air to operate?

A. MaxBlend Lite
B. MaxBlend 2
C. BlenderBuddy 2
D. MaxVenturi

Answer: D
Source: Technical Training, Page 11 – uses ambient air without piped air 



8. What is a key function of the MaxVenturi inlet filter?

A. Regulates oxygen concentration
B. Stops outside pollutants entering the device
C. Measures flow rate
D. Reduces gas pressure

Answer: B
Source: Introduction to Maxtec Blenders, Page 3 – inlet filter stops pollutants entering device 



9. What is the main purpose of the MaxVenturi muffler?

A. Increase flow rate
B. Reduce oxygen consumption
C. Reduce noise during high flow therapy
D. Improve gas mixing accuracy

Answer: C
Source: Introduction to Maxtec Blenders, Page 3 – muffler reduces noise for patient comfort 



10. What is the function of the flow diverter used with Maxtec devices?

A. To increase oxygen concentration
B. To allow gas to escape and prevent pressure build-up
C. To filter incoming air
D. To control flow rate

Answer: B
Source: Technical Training, Page 14 – bleed hole allows gas to escape and prevents pressure build-up

